# Victorian Data Supply Report

## Dataset

Dataset name:		gmw_dc.shp

Theme:			Drainage Catchments

Dataset Type:		Spatial

Description:	This dataset contains delineated drainage areas for the Goulburn-Murray Water Authority region within the state of Victoria. There are a total of 23 drainage catchment areas, which spatially are closely associated with the irrigation areas of the GMWA area. There are no drainage catchment areas delineated for the Torrumbarry or Pyramid Hill Irrigation areas as the drainage in these areas is all connected i.e. one drainage catchment for these areas.

Dataset use:		For use in the rural drainage activities and administrative definition of the drainage areas.

## Custodians

Goulburn-Murray Water Authority

### Data Quality

Lineage:		SKM supplied the boundaries in digital format.

Data capture method:	Supplied as a pre-existing digital dataset in a CAD format.

Data capture scale:	Assume 1:25,000

Completeness:		Dataset is complete except for the Torrumbarry / Pyramid Hill regions

Positional Accuracy:	unknown

Attribute Accuracy:	100 percent

Consistency:	Data is topologically consistent. Spatial data structure has been checked in ArcView to ensure spatial data integrity. All polygons are closed.

Comments:	There are no drainage catchment areas delineated for the Torrumbarry or Pyramid Hill Irrigation areas as the drainage in these areas is all connected i.e. one drainage catchment for these areas.

#### Geographic Extent

Extent Name: Goulburn-Murray Water Authority

Min Easting:	143.30			Min Northing:		-37.70

Max Easting:	148.25			Max Northing:		-35.10

##### Projection/Datum

Data was supplied in AMG Zone 55 (AGD) and translated to WGS84 (GDA94) using the ESRI Projection Utility Wizard (which is part of the ArcView software package) to meet the requirements of the MDBC IIGIS specifications.

The parameters in the Projection Utility used were:

Input Coordinate System Name:

Type: Projected

Name: AGD_1966_AMG_Zone 54(20255)

Units: Meter (9001)

Input Coordinate System Parameters:

unchanged from defaults

Input Coordinate System Datum:

Geographic Transformation: AGD_1966_To_WGS_1984

Output Coordinate System Name:

Type: Geographic

Name: GCS_WGS_1984(4326)

Units: Degree (9102)

Output Coordinate System Parameters:

unchanged from defaults

Output Coordinate System Datum:

unchanged from defaults

## Status

Last Update:		July 2001

Update frequency:	Irregular

Maintenance method:	unknown

Progress:		Complete

Restrictions:		As per agreement between MDBC and Rural Water Authorities

### Specifications

Data format:		Version 7.3 of the MDBC IIGIS

Format Type:		ESRI Shapefile

Data Type:		Polygon

Available format:	ESRI Shapefile

Data Size:	600 KB

#### Contacts

Name:		Paul Kenny

Organisation: 	Goulburn-Murray Water Authority

Position:		Team leader, Distribution Assets, Tatura

Victorian Coordinator MDBC IIGIS

Address: 	40 Casey Street,

Tatura, Victoria 3616

Mail Address:	PO Box 165

Tatura, Victoria 3616

Telephone    (03) 58335500

Facsimile      (03) 58335501

Electronic Mail Address – Paulk@g-mwater.com.au