Hydrogeology of the Great Artesian Basin

METADATA

RANSLEY, T., Feitz, A., RADKE, B., RANSOM, G., BELL, J., stewart, G.

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

The data referred to in this document can be obtained from Geoscience Australia at www.ga.gov.au

Version: 01

# Hydrogeology

## Great Artesian Basin – Hydrogeology

| Title | Great Artesian Basin - Hydrogeology |
| --- | --- |
| Record Id. in GA | Geocat record number: 81672 |
| Abstract | The GAB hydrogeological boundary follows the geological margins of its four component sub-basins - the Eromanga, Carpentaria, Surat and Clarence-Moreton sub-basins – except where the water table indicates groundwater divides within these regions that delineate externally-draining regions. In some regions, this drainage divide changes geographically through the GAB sequence. In such areas as the GAB boundary within the Clarence-Moreton Basin, the broadest hydrogeological extent has been adopted as the GAB boundary. This dataset documents a basin-wide hydrostratigraphy of the GAB comprising five major aquifers, four intervening aquitards, and the cover of these units by Cenozoic rocks and sediments. The extensive Quaternary sediment cover is considered permeable and not an impediment to the recharge of underlying aquifers. However, the underlying Paleogene-Neogene rocks and sediments have highly variable hydrogeological character that can influence deeper connectivity. In this map individual aquifers and aquitards within the Paleogene-Neogene sequence have not been differentiated. There are two data layers in the Great Artesian Basin – Hydrogeology map data A: Hydrogeological unit boundaries B: GAB Boundary This is a regional interpretation for mapping at approximately 1:1 000 000 to produce a broad scale overview, and examination of small areas by collecting extra data is most likely to produce results that differ from this regional interpretation. This dataset and associated metadata can be obtained from www.ga.gov.au, using catalogue number 81672. REFERENCES: Ransley, T., Radke, B., Feitz, A., Kellett, J., Owens, R., Bell, J. and Stewart, G., 2014. Hydrogeological Atlas the Great Artesian Basin. Geoscience Australia. Canberra. [available from www.ga.gov.au using catalogue number 79790] Habermehl, M. A. and J. E. Lau (1997). Hydrogeology of the Great Artesian Basin Australia (Map at scale 1:2,500,000). Canberra, Australian Geological Survey Organisation |
| Lineage | SOURCE DATA: Data was obtained from a variety of sources, as listed below: - Hydrogeology of the Great Artesian Basin Australia (Habermehl & Lau, 1997) - Scanned 1:250,000 scale Geological map series. Geoscience Australia 2010, Canberra. - Surface Geology of Australia, 1:1 000 000 scale, 2012 edition. METHOD: Formation Extent Extent lines were adjusted from the Hydrogeology of the Great Artesian Basin Australia dataset (Habermehl & Lau, 1997) using geology datasets (see data sources) to envelop all intercepts of the Hydrostratigraphic unit. This produced some varied and irregular shapes, some patchy regions, and required some interpretation to establish the most likely extent boundary. GAB Boundary The GAB boundary has been modified from the Great Artesian Basin Water Resource Assessment (GABWRA) project. Refinements in the boundary have been made in the Clarence-Morton, the Cape York, and the removal of the Laura Basin. QAQC: Data has undergone a QAQC verification process in order to capture and repair attribute and geometric errors. SOFTWARE: All modifications/edits and geoprocessing were performed using ESRI ArcGIS 10 software. |
| Use Limitations | These datasets have been compiled or interpreted from existing and new data sets that vary in scale. They are intended to be used for broad, regional understanding of the basin and are not designed to be used at a local scale. Where existing data sets have been used we have attempted to correct any errors, however errors may remain. It has to be stressed that this generalised basin-wide concept is scale dependant, and may exaggerate the distinction between the superposed aquitards and aquifers. Although this hydrostratigraphy offers more accessible comprehension of the regional hydroarchitecture, the generalisation comes with the inherent dangers of simplification and apparent enhanced contrast of a complex system. For local hydrogeological study, such generalisations may not necessarily survive closer scrutiny. |
| Extent |  |
| Scale | 1:9,000,000 |
| Projection | Lambert conformal conic GDA 1994, with central meridian 134 degrees longitude, standard parallels at -18 and -36 degrees latitude. |